PLANNING COMMITTEE REPORT

| Application Number | 2017/0575/OUT |
| --- | --- |
| Date Received | 6th September 2017 |
| Date of Expiry | 30th October 2020 |
| Case Officer | Ray Deans |
| Ward | Portland |
| Ward Councillor | Councillor Brian Lohan |
| Committee Date | 7th June 2021 |

| Site Address: | Gregory Quarry, Off Nottingham Road, Mansfield |
| --- | --- |
| Proposal: | OUTLINE APPLICATION FOR RESIDENTIAL DEVELOPMENT OF UP TO 204 DWELLINGS (WITH ALL MATTERS RESERVED EXCEPT ACCESS) WITH ACCESS OFF QUARRY LANE AND ASSOCIATED OPEN SPACE, LANDSCAPING AND INFRASTRUCTURE WORKS |
| Applicant: | Mr Lee O'Connor |

RECOMMENDATION – REFUSE PERMISSION

DESCRIPTION OF PROPOSAL AND APPLICATION SITE

This is an outline planning application in respect of proposed residential development within Gregory Quarry with all matters reserved except for access. The site comprises primarily of a former stone quarry, now exhausted and inactive for over 10 years. The site is a former dimensional stone quarry which produced a material used predominantly locally called ‘Mansﬁeld White’.

The former quarry site is situated 1 mile south of Mansﬁeld town centre and is located 16 metres below local ground levels within a triangle between Quarry Lane to the north, Nottingham Road A60 to the east and a 1940’s/ 50’s housing development to the south-west. To the north of the site, is an area of land of grassland and trees that is accessible to the public, which is bound by Quarry Lane to the north and Gregory Quarry to the south, separated from the quarry by a disused railway embankment. A permissive right of way footpath runs along the embankment, and the River Maun runs through this area with another footpath running parallel, to the south of the river. Part of this land is Quarry Lane Nature Reserve which extends further, to either side, along to the River Maun.

To the north east of Gregory Quarry is Mansﬁeld Leisure Park along with Sainsbury's and Aldi supermarkets. Mansﬁeld Leisure Park includes retail development, restaurants and a cinema. The rear of the cinema backs onto the quarry, with the back of the supermarkets facing the site access off Nottingham Road.

The existing access road off Quarry Lane was historically the primary access route serving the quarry, as is evidenced by historic maps. The existing roadway runs through the Quarry Lane Nature Reserve via a bridge across the River Maun and through a short tunnel through the disused railway embankment. The access road off Quarry Lane was regularly used for large vehicle movements for many years and the bridge located across the River Maun would undoubtedly have been constructed solely for the purpose of accessing the quarry.

There is no public pedestrian route through Gregory Quarry between Nottingham Road and Quarry Lane. The only pedestrian routes linking the local community from Nottingham Road to the Quarry Lane Nature Reserve are an extended route along Sheepbridge Lane to the south or via the privately owned Mansﬁeld Retail Park to the north.

THE APPLICATION

The application relates to the proposed use and means of access only. Appearance, landscaping, layout and scale would be subject to subsequent reserved matters applications.

The applicant seeks to redevelop the brownﬁeld site for a residential development of 204 units with enhanced vehicle, pedestrian and cycle access.

The planning application is accompanied by the following supporting documents:

- Planning Supporting Statement

- Indicative Site Layout

- Site Access Proposals

- Flood Risk Assessment

- Landscape and Visual Impact Assessment

- Extended Phase 1 Habitat Survey and Ecological Appraisal

- Arboricultural Impact Statement and Assessment

- Geo-Environmental Risk Assessment and Geo-Environmental Ground Investigation

- Statement of Community Involvement (SCI)

- Transport Assessment / Statement

- Biodiversity Accounting Exercise

- Viability Assessment

The Planning, Design and Access Statement states that the separate Planning Statement provides a summary of the planning context for the site and applicable planning policies, although this was produced prior to the Council’s updated Local Plan being adopted in September 2020.

RELEVANT SITE HISTORY

The site is a former dimensional stone quarry which produced a material used predominantly locally called ‘Mansﬁeld White’.

The quarry continued to be worked until approximately 10-15 years ago by which time it was exhausted for stone extraction.

OBSERVATIONS RECEIVED

Throughout this report observations received in respect of each application are presented in summary form.  The full letters and consultation responses received, including details of any non-material planning observations, are available for inspection both prior to and at the meeting.

Anyone wishing to make further comments in relation to the application must ensure these are received by the Council by 12 noon on the last working day before the date of the Committee.

Statutory, Internal and Other Consultees

(2017) Nottinghamshire County Council - Local Highways Authority

Concerns over the ability to provide the required status of the access for emergency access. Conflicting information regarding emergency accesses – a legal opinion is that they must be adopted. It is noted that the proposals for removal of the tunnel, severing the permissive path known as Mansfield Way now include steps and a ramp to/from road level.  This is a matter that Nottingham County Council (NCC) Planning Policy comment on and the severance of the route as previously submitted was strongly objected to.

The Section to the east of the development access road would become inaccessible by the required maintenance vehicles as a result of the current proposals.

The HA remain of the view that accurate information in support of the viability assessment should be provided and this does not appear to be the case.  It is therefore considered that there is a significant risk that the development cannot provide the necessary access arrangements.

The following issues are challenged in the viability assessment:

- The access to the site – this now includes significant ramps and steps which are not included, but this may well be unacceptable to NCC in any case and require a bridge capable of taking maintenance vehicles.  The cost of the required Traffic Regulation Order has also not been included and whilst the report uses the estimate given in Appendix VII, the body of the report itself identifies that the box culvert used (costed at £275,000 would be more in the region of £650,000 (and this is considered still to be an underestimation).

- 80k has been allowed for offsite mitigation works. As £50k is required for the works to the traffic signals, as set out in the Highway Technical Note (HTN) this leaves £30k for the widening works on Quarry Lane, which have actually been costed at £553,100 without the required retaining structure, along with the proposed pedestrian refuge to allow safe pedestrian access to and from the bus stops on Nottingham Road costed at £8,100. £30k is clearly therefore a significant underassessment of costs.

- Commuted sums for the structures (bridge and retaining) and traffic calming have not been included and represent a significant cost.

The Local Highway Authority considers that the access costs are grossly underestimated, which could prejudice the viability of the site access, and result in unsatisfactory access arrangements.

(2021) Nottinghamshire County Council - Local Highways Authority

In terms of Mansfield Way, the source of the previous objection from NCC’s Policy team, the applicant is either suggesting that:

- NCC can use a vehicle not in their ownership to gain access via the ramps, which require land outside of the control of NCC or the applicant, to be built as shown - therefore access and maintenance issues are unclear

- Access is provided from the retail park – this land is also not within the applicant’s control and therefore cannot be guaranteed.

The proposals are prejudicial to the existing users of the Mansfield Way by requiring them to descend from this off-road route, cross a wide road in conflict with motorised vehicles and cyclists on the cycle route and then ascend either steps or a large ramp to return.  The principle of the proposal therefore appears to be in contravention of Paragraph 110 of the National Planning Policy Framework (NPPF) and a ‘like for like’ facility should have been provided in the form of a bridge (that is also capable of carrying maintenance vehicles).

The development constrains access significantly and appears to require a specific tracked vehicle that does not form part of the NCC fleet and therefore it would be necessary for the developer to provide the smaller maintenance vehicle suggested.  They are thought to cost in the region of £250k.

There is a suggestion in the appellant’s Transport Note that the development could fund retractable bollards to enable access from the retail park.  This would be the most easily workable solution.  However, this would involve crossing land which is not in the applicant’s control and if that option were to be pursued the applicant/developer would have to arrange a suitable Deed of Grant with all relevant adjacent land owners securing that access for NCC, so that there would be certainty that it would remain open in the future.  However, this introduces further uncertainty in respect of the delivery of the required accesses and another ransom potential and financial uncertainty.

Emergency Access

Due to the only access to the site being across a watercourse via a new bridge and also in consideration of the distance of the development from public transport, the Highway Authority (HA) requires the pedestrian/cycle route combined with an ‘emergency access’ linking the site to Nottingham Road.  Emergency accesses are not normally acceptable for numerous reasons, but without it, the proposed development is considered to be unacceptable, as it risks being completely cut off should the bridge be closed and this would also not address the core requirements of the NPPF in terms of sustainability - with the link, the development is around 400m distance from the nearest bus stop and without it 750 metres apart.

The provision of an emergency access has its disadvantages, as in addition to emergency accesses not normally being acceptable (due to enforcement problems arising from their misuse, potential difficulties that could be encountered by the emergency services, maintenance issues and vandalism of access-control equipment, and general crime and anti-social behaviour problems) the applicant would not be in control of all of the land required to provide it.

Furthermore, the restricted ownership would result in private rights of way only, available to future residents of the site - the route could not be adopted nor accessible to anyone other than residents of the site (enforced by the proposed access control measures) and is not considered of community benefit.  This conflicts with the Nottinghamshire Highway Design Guide which requires emergency accesses to be of strategic benefit for pedestrians and cyclists.  Officers are also unaware of any consultation with the Emergency Services on the matter.

It is noted that there is also a small section of land adjacent to the highway on Nottingham Road that was on first inspection unregistered but now appears to have a claim against it.  It is not known who is claiming the land, but this could also present a ransom situation by a fourth party.

Quarry Lane Access

This is a complex engineering project involving structural retaining walls and significant amounts of earthworks.  As the land drops away steeply to the south of Quarry Lane, the retaining wall will be large and visually significant.  We believe the costs of this in the viability assessment are significantly underestimated.  Given concerns in respect of the development’s viability and need of the structure to make not only the development, but the construction access acceptable, we would require this to be built pre-commencement.

Access Road

Many of the concerns regarding viability are based on this unknown cost of acquisition of land/rights over land owned by 3 separate landowners.  Should development commence prior to this being built, there is a risk that houses could be built without suitable access being able to be achieved.

Furthermore, it is required to provide suitable construction access to the site, as this will not be allowed from the A60 (along the emergency access route).

Accordingly, we would require the route to be provided in full prior to commencement.

Offsite Works

The cost of the required offsite works is £50,000 for the required amendments to the traffic signals at Sheepbridge Lane addressing capacity issues and £8,100 for the pedestrian refuge on Nottingham Road, to allow safe pedestrian access to and from the southbound bus stop.  These figures have been included within the viability assessment.

Viability (update)

Officers have reviewed the letter from the developer’s Chartered Surveyors, contained within the Technical Note, dated 23 February 2021, and accept that some figures initially highlighted as not having been included in the viability assessment are actually included.  However, having now reviewed the viability assessment in more detail, we retain significant concerns in respect of a number of issues.

The figures quoted in the above letter do not correlate with the figures quoted in the Viability Assessment (VA), which is uploaded on to the Local Planning Authority’s (LPA) website.  However, as the valuation of ransom strips are often based upon percentage of profit, the uncertainty and our concerns remain.

Furthermore, whilst the letter differs, Section 11 of the Viability Assessment states that, “even without affordable housing, the viability is just acceptable”.

The Technical Note states that “any viability concerns that NCC Highways have do not impinge on the assessment of the merits of the proposed scheme and its access arrangements and should not be a reason to object to any proposal”.  It is the experience of NCC that, should planning permission be granted with conditions and/or obligations, these may prove to be unenforceable during the reserved matters stage due to the uncertainty of costs which inform the viability assessment.  We draw the LPA’s attention to the risks associated with basing the viability assessment upon uncertain costs, which could result in the inability to afford the access works required for this site. It is for this specific reason that the HA has paid particular attention to the viability study presented to the LPA.

It is not clear how the figure for acquisition of rights to access the development has been ascertained but the acquisition of rights to access land assumed at £500k is potentially unrealistic.  There are 3 (and now potentially 4, dependent on the owner of the land required at the Nottingham Road end of the emergency access) distinct landowners over which the access transgresses and requires the land to be built on and dedicated as highway and each will view it as a ransom opportunity.  There is no one size fits all for this type of ransoming valuation. In some cases, this has been determined as 30%-50% of the uplift value. Therefore, if the value of the development site is, as suggested in the VA, £8m, £2.6m - £4m may need to be paid to other land owners.

It is also considered highly likely that the numbers of dwellings considered within the viability assessment will have to be reduced to accommodate a suitable, safe and adoptable road layout.

The Technical Note suggests that commuted sums are covered in the valuation, but these have not been referred to in the viability report and there are no calculations.  Commuted sums will be required for the bridge, retaining structures, additional carriageway and potentially the ramp and steps to Mansfield Way.

Currently, the value of these are unknown until the details of the scheme are finalised, but these are likely to be significant sums.

In addition to the three unknown elements above, which are considered be significant, there are a number of specific costs which have either been underestimated or not included in the valuation.

Whilst the viability assessment appears to include the development roads (“access roads), the figure presented appears to be a significant undervaluation.  The figure estimated below is for the roads and footways and does not include the private drives and car parking as included in the viability assessment.  However, the submitted estimate does not allow Officers to make a more accurate assessment of the underestimation.   The estimate for the adoptable roads/footways is £750,000.  In the interests of being reasonable and to include for the private drives, we have suggested an underestimate of £500,000

| Additional specific underestimations / costs not considered within the viability assessment: | Estimate |
| --- | --- |
| Internal roads (under-estimation) Supervision fees for the above | £500,000 £52,500 |
| Supervision costs for the Section 278 works and access road (based on the estimates presented but supervision fees are calculated based on NCC rates which are likely to be higher). | £230,000 |
| Cost of ramp and step system for the Mansfield Way proposals | £65,000 |
| Underestimation of retaining walls on Quarry Lane. £125/linear metre has been allowed but a more realistic figure is £800 - £1k/linear metre | £100,000 |
| Cost of Traffic Regulation Order / Travel Plan Incentives & Monitoring Fee not included | £20,000 |
| Additional lengths of road restraint system required plus £8k for frangible ends | £13,500 |
| Legal fees for Section 278 and Section 38 Agreements | £10,000 |
| Engineering fees for detailed design | £10,000 |
| S106 contribution for provision of maintenance vehicle required for access to Mansfield Way. | £250,000 |
| TOTAL | £1,250,500 |

Nottinghamshire County Council (Planning Policy)

It is considered that the following Local Plan policies are relevant to this application.

NE2 (Green infrastructure)
NE7 (Biodiversity)
NE8 (Protection of designated biodiversity and geodiversity sites)

NE13
CC3 (Flood Risk)
CC4 (Impact of development on water)

2017 Ecology/Biodiversity

Need to protect Local Nature Reserves (LNRs) and sites of ecological, geological and geomorphological value and the network of such sites.  Policy NE8 applies to sites which have been designated due to their biological or geodiversity value.  The designations local wildlife sites (LWSs) are considered sites of ecological value and local geological sites (LGSs) are considered sites of geological and geomorphological value.  Geology is considered below.

Local Nature Reserves are designated under the National Parks and Access to the Countryside Act (1949); it should be clarified if this Act provides any additional protection to LNRs above that which is covered under the present planning system and if any local by-laws apply.

Policy NE13 sets out that planning permission will not be granted for developments which would damage such sites unless the reasons for the development clearly outweigh the intrinsic value of the site or the contribution it makes to the local network of such sites.

The access for the proposed development passes through the LNR and two LWSs and will impact on these through some loss of designated area, habitat and amenity and other impacts during the construction stage.

Where development would be deemed appropriate, Policy NE14 seeks to ensure that important features are retained and protected where possible and provisions made for the future management of such features. Where loss is unavoidable compensatory measures are sought.  It is recommended that substantial weight be given to Policies NE13 and NE14; together they are consistent with paragraph 113 of the NPPF.

Policy NE8 sets out that planning permission for development that impact on sites of importance for biodiversity (such as LNRs and LWSs) will only be granted where:

- The benefit of the development outweighs the significance of the protected site and its position in the hierarchy, and harm caused;

- It can be demonstrated that avoidance and mitigation has been followed in accordance with Policy NE7 and relevant legislation;

- They are accompanied by a relevant assessment to demonstrate the impact of development upon the designated site.

Policy NE7 addresses the protection and impact on protected species and other species covered by the Wildlife and Countryside Act 1981 (as amended) and other relevant wildlife legislation. The Preliminary Ecological Assessment assesses impacts on protected species through key wildlife legislation and habitats and species of principle importance as covered under the Natural Environment and Rural Communities Act (2006).  The surveys and records used to assess protected species were carried out in 2015 - more recent survey work would be required in order to fully assess the impacts on wildlife and the effectiveness of development to adequately address these.

It is noted that the Biodiversity Accounting Exercise that has been carried out has identified that there will be a net reduction in biodiversity features and additional measures will be required to reduce impacts and lead to a net gain in biodiversity (Planning Statement paragraph 9.9 to 9.11).  However, at this stage it is unclear what measures are proposed or could be provided.

In accordance with the sequential approach set out in Policy NE8 of the Local Plan and paragraph 118 of the NPPF further information is required to demonstrate:

- That the harm cannot be avoided by considering alternative schemes designs (especially access solutions);

- What mitigation could be provided if the harm cannot be avoided; or

- The compensation that would be provided.

This would address the LNR, LWS and any impacts on protected species and habitats and species of principle importance.  It should also be noted that whilst the Nottinghamshire Wildlife Trust is a knowledgeable resource, other bodies such as the Environment Agency should be consulted to inform assessed impacts on and potential avoidance and mitigation of water voles, otters and other aquatic species.

Geology

In relation to the designation of the former quarry area as a Local Geological Site it is noted that the applicant has corresponded with Nottinghamshire Biological and Geological Records Centre (NGBRC) who identify such sites.  It is reported in paragraph 9.4 of the Planning Statement that NGBRC have confirmed that the features that led to the designation of the site have been lost or are no longer visible; emails are provided to confirm this.  As the site is no longer considered to meet the criteria for designation as an LGS, Policies NE13 and NE8 are not relevant to this application in relation to geology/geomorphology.  These policies are relevant, however, in relation to the local wildlife sites and the local nature reserve.

Green Infrastructure Corridor

The site forms part of the River Maun Valley identified by Policy M12 (A) of the 1998 Local Plan.  The Policy states that planning permission will not be granted for developments that would prevent the implementation or lead to the subsequent loss of strategic routes for walkers, cyclists and horse riders, including areas of informal space, unless acceptable alternatives are provided along the River Maun Valley.  It recommended that substantial weight be given M12 (A) as it is consistent with the approach in the NPPF. It is not clear through the proposed development, how the proposal will impact on the strategic route and associated informal space.

Policy NE2 of the (emerging) Mansfield Local Plan 2020 and the associated policies map sets out areas of strategic green infrastructure and states that planning permission will be granted within or adjacent to areas of strategic green infrastructure where it enhances its multi-functional role in providing an accessible, functional, healthy and robust natural environment.  These policies are in-line with the approach taken in the NPPF (paragraphs 109 and 114) and NPPG on green infrastructure which emphases the need to protect and enhance its multi-functional benefits and that these should be well planned, designed and maintained.

These policies also need to be considered alongside those on ecology. The proposal is within an identified strategic green infrastructure network which provides ecological, recreational and climate change benefits in relation to the River Maun and adjacent woodland.  It is unclear from the information submitted the degree of impact on the strategic Green Infrastructure (GI) network and, if and how impacts could be appropriately mitigated and enhancements provided.  It is recommended that further work is carried out to address this in consultation with Mansfield District Council, Environment Agency and any other relevant parties.

Flooding and Water Quality

It is recognised by the Council and the Environment Agency that this and downstream sections of the River Maun has experienced high levels of silting which has exacerbated flooding across the A60 at Field Mill Dam.  Although the Flood Risk Study and Sustainable Drainage System (SuDS) Statement submitted with the application address flood risk within the proposed site and adjacent existing dwellings, they do not appear to address potential impacts from the proposed development regarding silting up, other pollution sources during and post construction further downstream and subsequent impacts on flooding.  It is not clear from the Phase I Environmental Assessment and SuDS statement what impacts there may be on water quality.

The Environment Agency and the County Council (as lead flood risk authority) should be consulted on how the SuDS proposals will help to reduce contamination of the River Maun and not increase flooding further downstream and any contributions to existing infrastructure (i.e. strategic green infrastructure and culverts) required to address these issues.  It is unclear as to what constitutes ‘best practice Pollution Prevention Guidelines’ and how these should be integrated, as referred to in the Preliminary Ecological Assessment.  The Mansfield District Council (MDC) Strategic Flood Risk Assessment (2008) and its addendum (2016) provide further information.

The scale and nature of the impact of the access to the proposed development site on the Local Nature Reserve, local wildlife sites, strategic green infrastructure network is unclear.  The work done to avoid, minimise and compensate this impact should be identified.

Summary

It is recommended that additional information is provided in the form of maps and plans showing the important ecology/biodiversity features and how these are impacted by the proposed access.

Evidence should also be provided to set out how the impact on these has been avoided where possible, mitigated or compensated.  Information should also be sought regarding the impact on the wider GI corridor and the potential impact on the water quality of the River Maun.

Once the scale and nature of any impacts have been identified these can then be weighed against the benefit of providing housing on a site within the existing settlement boundary of Mansfield and addressing the anti-social behaviour issues that exist.

Planning Policy object to the proposals until these matters are addressed.

2018

Noted that there is an amendment to the access road from the north, whereby a bridge is now proposed to take the Mansfield Way over the access road. The Mansfield Way is an NCC Green Spaces site managed by Nottinghamshire County Council and as a result, NCC has the following comments:

- It is not clear who will be responsible for maintenance of the proposed new bridge. This needs to be clarified, and any ongoing maintenance costs will need to be provided via a commuted sum in the event that it is expected that NCC Green Spaces will take on responsibility for the bridge.

- It appears that the bridge is not located on the current line of the Mansfield Way path, so the developer will need to provide additional sections of surface path, at their own cost and to our specification, to link to the bridge.

- The bridge provides an opportunity to deliver modest ecological benefits, so NCC would request that it is designed to include incorporated bat boxes (12no.), with details to be agreed

- It is not clear if there is intended to be access from the proposed development, onto the Mansfield Way. If there is, then a commuted sum will need to be provided for additional maintenance works that will be required as a result of increased public use, and there is a need to calculate this so it can be included in the S106 agreement.

- In addition, NCC requests details of the standoff between the boundary of the Mansfield Way and the nearest property, to determine if there is likely to be an increased requirement for the management of trees along the Mansfield Way with regards to their potential to fall into the gardens of adjacent properties.

2020 Ecological Impact

The County Council does not have the capacity to comment on the ecological impacts of the proposal in any great detail; in any event, whilst revised ecological information is referred to in the cover letter, no such information appears on the MDC website. However, generally speaking, the proposal will evidently result in the loss of a large area of green space (both in the former quarry and along the access road), with very little mitigation provided, given the level of built development. I would suggest that a Biodiversity Net Gain calculation should be carried out to determine the losses/gains to onsite habitat, and to identify the level of mitigation required (which might well have to be delivered offsite).

Impact on the Mansfield Way

The proposed access road would cause considerable damage to both the Quarry Lane LNR/River Maun corridor, and the Mansfield Way, severing a green corridor and introducing a considerable amount of disturbance (e.g. significant numbers of vehicle movements). Of particular concern to the County Council is the impact on the Mansfield Way, a section of former railway embankment which forms the northern boundary of the residential development and is managed by NCC Green Spaces. The accompanying plan shows the extent of NCC Green Spaces interest at this location, edged in purple. Land Registry titles can also be provided if necessary.

The section of the Mansfield Way at Quarry Lane is a 0.5-mile-long permissive route linking the Mansfield Leisure Park off the A60 Nottingham Road with a public footpath running south-west from Mansfield town centre adjacent to the Robin Hood Railway Line.  This route includes the historic Drury Dam Viaduct, which supplies a valuable off-road crossing off Quarry Lane.  When extended to include the adjoining footpath the route also provides an off-road crossing of Sheepbridge Lane.  The access road to the proposed development as shown on Site Plan Proposed revision P08 dated 14/08/20 would isolate the section of path at the Mansfield Leisure Park (western) end from the Viaduct and both off-road crossings (western end).  A significant diversion though Quarry Lane LNR would be required to cross the access road and then follow a further diversion to gain access to the Viaduct.  Given that the Mansfield Way in this area is a valued and well-used off-road route this is not acceptable.  It should be noted that the disused railway line forming the Mansfield Way was purchased by Nottinghamshire County Council in 1988/89, meaning that permissive access has been provided for somewhere in the region of 30 years.

Vehicular access to this section of the Mansfield Way for maintenance purposes is via Quarry Lane and the Drury’s Dam Viaduct.  The proposals would isolate the section of path east of the proposed access road from vehicular access, making maintenance impossible without crossing neighbouring landowners’ property – which we do not have permission to so.  This is particularly concerning given the mature tree stock typical of the area, which have in recent years required work needing vehicular access.

On this basis, the County Council must strongly object to the proposals, given the unacceptable impact on the Council’s site. The County Council would note that no approach has been received from the developer or their Agent in relation to their need to cross our land.

NCC Lead Local Flood authority

No objection to the proposal, but a detailed surface water drainage scheme should be conditioned.

Natural England

Statutory nature conservation sites – no objection

Based upon the information provided, Natural England advises the Council that the proposal is unlikely to affect any statutorily protected sites or landscapes.

Protected species

We have not assessed this application and associated documents for impacts on protected species.

Natural England has published Standing Advice on protected species.

You should apply our Standing Advice to this application as it is a material consideration in the determination of applications in the same way as any individual response received from Natural England following consultation.

The Standing Advice should not be treated as giving any indication or providing any assurance in respect of European Protected Species (EPS) that the proposed development is unlikely to affect the EPS present on the site; nor should it be interpreted as meaning that Natural England has reached any views as to whether a licence is needed (which is the developer’s responsibility) or may be granted.

Mansfield District Council

MDC Policy

Priority and protected habitats and species within and supported by the site will still need to be protected and enhanced as per Policies NE2, IN2, NE14 and NE16 of the Mansfield Local Plan 2020.

The proposed site is part of the strategic green infrastructure network as per the local plan and policies map. Thus, Policy IN2 would still apply.

NE2 applies in relation to addressing Natural Environment and Rural Communities act 2006 (NERC) Section 41 habitats and species and important landscape features, protected species, impacts on adjacent LNR and LWS and biodiversity net gains, etc.  The NPPF 2019 sets out the need to support net gains in biodiversity. The application would need to sufficiently address these as they are material considerations. Policy IN6 applies as there is a nearby local green space designation.  As well as IN4 on open space within new development.

The adjacent LWS are within MDC ownership.

MDC Conservation

Any altered access should include a repositioned replicated stone boundary wall ans semi-circular coping feature and all repairs in a like for like basis.  The loss of a section of the viaduct would be a concern and reference would be to retain it. Natural building materials should be used.

MDC Environmental Health

No objections subject to conditions regarding noise, environment, hours and contaminated land.

Heritage England

Need to take into account the importance of securing access to any remaining Mansfield White and Red Stine resources, particularly as it is the last remaining source of Mansfield White stone in the County.

Nottinghamshire Wildlife Trust (NWT)

Majority of the survey work was undertaken in 2015 and is currently out of date. As development would not commence in the near future, it would be necessary for further detailed survey to be undertaken prior to commencement to update this information and to undertake missing/inadequate survey work from 2015. A small additional area to the north was subject to a walkover survey in 2017 and it is not clear whether the protected species survey work from 2015 covered this area. If detailed, up to date surveys do not cover the whole application area and surrounding zone of influence, there is a risk that ecological impacts may not have been adequately assessed. The NPPF states in paragraph 165 that planning decisions should be based on up-to-date environmental information.

The PEA identifies that the site is well connected to the surrounding area and that it contains three designated sites, Quarry Lane Local Nature Reserve (LNR), River Maun Local Wildlife Site (LWS) and Maun Woodland and Scrub LWS. However, upon examination of the latest site plan, all three sites as well as the ecological connectivity are under threat from the proposed access road. The River Maun and associated habitats clearly provide an ecological corridor function which would be severely impacted by the current proposal. This is contrary to a number of policies within the draft Mansfield District Local Plan (detailed further below):

- S1 Sustainable Development

- NE2 Green Infrastructure

- NE5 Protection of local green space NE5

- NE7 Biodiversity and geodiversity

- NE 8 Protection of designated biodiversity and geodiversity sites

The proposed access road which bisects the River Maun, Quarry Lane LNR and both LWS’s appears to be in direct conflict with this approach. Thus, we consider the proposed site access from the north to be unacceptable on both ecological and policy grounds.

Many of these habitats are priority Biodiversity Action Plans (BAP) habitats/Habitats of Principal Importance under Sn 41 of the NERC Act. The mitigation hierarchy requires that rigorous effort should be made to first avoid impacts on such habitats before considering mitigation or compensation, this approach does not appear to have been taken in this case. Paragraph 118 of the NPPF is clear in this regard.

If it can be demonstrated that the need for this development outweighs the requirement to avoid impacts on the LWS, then it would be appropriate to consider mitigation and compensation, however NWT do not believe that this need has been demonstrated.

If the LPA considers that the need for the development has been adequately and robustly demonstrated, then the LPA should seek confirmation from the applicant at this stage that compensatory habitat could be provided to ensure no net loss of biodiversity. Without this assurance, this proposal should be refused.

Surveys recorded the presence of amphibians including smooth newts, frogs and common toads (a NERC 2006 Sn 41 Species of Principal Importance). The PEA states that there are likely to be ‘minimal alternative breeding opportunities’ within the surrounding habitat. This seems to indicate that the loss of standing water on site would cause considerable impact on these species. Proposed mitigation in the form of pond creation would only be sufficient if properly designed, installed, managed and monitored over the long term by suitably qualified ecologists. The scale, habitat quality and timing of the proposed mitigation would need to demonstrate that there would be no adverse impact on this toad population, NWT do not consider that this information has been provided by the Applicant.

The habitat assessment for bats concluded that the site provides ‘High’ habitat quality. Therefore, in accordance with the stated guidelines (Collins, 2016) transect and static bat surveys undertaken are not considered adequate to fully assess the importance of the site for bats. A total of three transect surveys were carried out in June, July and September along with a single point of static monitoring on three occasions. According to Collins (2016), where a site is assessed as having ‘High’ habitat quality, the guideline is for up to two transect surveys per month (April to October) and three static monitoring locations per transect, again monthly (April to October). Along with additional transect surveys, there are a number of trees identified with moderate potential for bat roosting which require further survey work to be undertaken before this application is determined.

Paragraph 99 of Government Circular 1/2005 (also known as ODPM Circular 06/2005) (which accompanied PPS9, but remains in force), states that:

‘It is essential that the presence or otherwise of protected species, and the extent that they may be affected by the proposed development, is established before the planning permission is granted, otherwise all relevant material considerations may not have been addressed in making the decision. The need to ensure ecological surveys are carried out should therefore only be left to coverage under planning conditions in exceptional circumstances, with the result that the surveys are carried out after planning permission has been granted.’

Once the results and assessment of the above surveys are available, the applicant should demonstrate to the LPA how adverse impacts could be avoided, mitigated or compensated. This information should be submitted to the LPA with clear assurance on how these would be delivered and guaranteed in perpetuity.

Potential nectar sources and larval food plants were found in abundance on the proposed development site. The majority of suitable habitat would be destroyed to facilitate the development as proposed, again this habitat should be retained in situ. If this impact could not be avoided the applicant should be able to demonstrate at this stage how and where replacement habitat could be incorporated into the site layout plan and how it would be secured for these Sn 41 species in perpetuity.

A thriving water vole population has been identified on the River Maun in the area of the proposed new access road/bridge. The development works would impact directly on water vole habitat during construction, potentially fragmenting the population. Post-construction, it is considered highly likely that impacts would continue due to disturbance from noise, additional traffic and lighting for example. In addition to severance of the valuable ecological corridor formed by the River Maun and Quarry Lane LNR, the access road is considered unacceptable due to impact on water vole, a species protected under the Wildlife and Countryside Act 1981 (as amended).

Section 4.3.5 of the PEA confirms that there is habitat suitable for otters and that foraging and commuting cannot be ruled out. NWT is aware of reports of otter on the River Maun to the north of the site. Otters, especially young animals dispersing from natal areas, are prone to being killed by road collisions so siting the new access road across the River Maun is unacceptable. As a Protected Species under European Law (Habitats Directive Annexe IV “Animal Species of Conservation Interest in Need of Strict Protection”) and UK Law (Wildlife and Countryside Act 1981 Schedule 5) it is essential that as the Planning Authority you are certain that this proposed development would not harm otters.

Breeding and wintering bird surveys were not undertaken, however the PEA identified suitable nesting habitat for a wide variety of bird species. Birds of particular interest known to NWT include kingfisher, spotted flycatcher, woodcock, song thrush, mistle thrush, starling, dunnock, and mallard – all listed as birds of Conservation Concern (Red and Amber lists). For a proposed development of this scale on a site with such a wide variety of habitats, we would expect that proper breeding and wintering birds’ surveys would have been undertaken in order to fully assess the potential impact on protected and priority bird species. This work should be undertaken before the application is determined as without it you are not, as LPA, in possession of the full facts regarding the value of the site.

Inaccessible areas meant that a full badger survey of the site was not undertaken. Whilst there was no evidence in the areas surrounding the dense scrub, the presence of badgers cannot be conclusively discounted. Further survey effort in this regard would be required, as a minimum at Reserved Matters stage.

In summary, the surveys undertaken were not as required under either the Chartered Institute of Ecology and Environmental Management (CIEEM), Ecological Impact Assessment (EcIA) Guidelines 2018 or BS 42020 Biodiversity - Code of Practice for Planning and Development, and thus are not adequate to provide sufficient details to inform your determination of this application.

This proposed development would cause major adverse impacts on habitats and species of conservation importance and little effort appears to have been made to avoid these impacts, contrary to the requirements of the NPPF paragraph 118.

The PEA is clear that works to facilitate the proposed access road would impact woodland and riparian habitat. Compensation for loss of riparian habitat is proposed, however no information on extent or location is provided.

The PEA suggests that all LWS fall within the LNR boundary and therefore compensation is only required for the LNR. Examination of a map of LWSs and LNR shows that parts of Maun Woodland and Scrub are not within the LNR and separate compensation for habitat loss would therefore be required.

Considering the proposed site plan, it appears that the ecologist’s recommendation for retention and protection of all woodland (5.1.4) would be unlikely and therefore mitigation and/or compensation should be proposed at this stage in the application to clearly demonstrate how ‘no nett loss’ of biodiversity would be achieved.

(5.1.6). We query how and where this would be achieved on a site where the current plan largely relegates wildlife habitat to the extreme boundaries of the development and takes little account of the scale and diversity of important species present on the site.

We welcome that a Biodiversity Impact Calculator – Habitats has been made available; however, we are extremely disappointed to see that it currently shows a net biodiversity loss of almost 72%. The PEA states that the metrics will ‘inform a scheme that will attempt to result in a “no net loss” of biodiversity at the site, and ideally provide a net gain for wildlife where feasible. That this has not been provided with the application documents is unacceptable and we cannot see how the application can be determined in its absence. The LPA are not in a position to assess how the proposal meets requirements within the NPPF as well as local policy and should therefore refuse the application.

This application clearly impacts on the River Maun and associated habitats. Indeed, we note that the Sustainable Drainage Statement refers to discharge of surface water into the Maun. With this in mind, we expect the LPA to request a Water Framework Directive (WFD) Assessment be included, before this application is determined.

Local authorities have statutory duties to deliver Water Framework Directive (Directive 2000/60/EC) objectives – this EU legislation is implemented in England and Wales though The Water Environment (Water Framework Directive) (England and Wales) Regulations 2003. Article 4(7) of the WFD sets the criteria that must be applied when determining development applications. If a proposal is likely to lead to deterioration in the overall status of a waterbody, or prevent future attainment of good status, it must meet the following tests, which can be addressed through a WFD Assessment:

• Has all practicable mitigation been included?

• Are there reasons of overriding public interest?

• Do the benefits to human health, safety or sustainable development outweigh benefits to environment and society?

• Are better environmental options not technically feasible or cost proportionate?

The proposal has the potential to have a negative impact on the water environment and no WFD assessment has been provided which could help to address this issue.

“Policy NE2 Green infrastructure

Planning permission will be granted within or adjacent to areas of strategic green infrastructure (as shown on the policies map) where it enhances its role in providing an accessible, functional, healthy and robust natural environment. Where development would result in the loss of any individual green infrastructure asset, replacement provision which is of equivalent or greater value will be required in order to improve the overall strategic value of the GI network. Planning permission will be granted for major developments which provide a combination of GI benefits* and clearly show how they address:

a. key principles and actions within the GI and Biodiversity Supplementary Planning Document**, where applicable

b. the integration of public open space and other amenity areas within green corridors and other access links

c. how the GI will be managed and by whom

d. how the creation and enhancement of GI (on / adjacent to the development site) contributes to combined environmental, community and / or regeneration benefits e. how local communities and appropriate organisations have been consulted in the design of new or enhanced GI.

*Benefits include health and wellbeing, transport and accessibility, climate change adaptation, enhancing heritage settings, regeneration, social cohesion and biodiversity.”

The site has been allocated as Strategic Green Infrastructure within the Local Plan – the proposal clearly does not meet the requirement to enhance its role in ‘providing an accessible, functional, healthy and robust natural environment’.

“Policy NE5 Protection of local green space

Planning permission will only be granted for developments on a site designated as local green space, provided that the development clearly enhances the area for the purposes it was designated. Developments proposed adjacent to a local green space will be granted planning permission if it would not harm the purpose for which the area was designated.”

Policy NE5(m)

The Scrape local green space lies within Quarry Lane LNR adjacent to the application site.

“Policy NE7 Biodiversity

Planning permission will be granted for developments which conserve, and where feasible provide net gains in, biodiversity by maximising opportunities to incorporate biodiversity enhancements across a landscape-scale, as it relates to the development. Where harm to the ecological network cannot be avoided, mitigation should be provided. If this is not possible, off-site compensation must be provided in order for the development to be permitted. All development proposals which affect ecological networks or protected species must be accompanied by an ecological assessment which demonstrates how the development will:

a. prioritise the protection and enhancement of priority or protected species, or priority or irreplaceable habitats

b. prioritise the protection and enhancement of landscape features of major importance for wildlife

c. where possible, avoid fragmentation of the ecological network

d. restore missing habitat links and landscape features through habitat creation and re-creation. Where this is not possible, buffer priority habitats and designated sites through the creation of complimentary habitats within landscaping and green space

e. address long-term sustainability of biodiversity through appropriate design and management plans

f. plan for the movement of wildlife within and off the proposed development site g. prioritise the retention / creation of habitats and landscape features in appropriate publicly accessible areas.

This policy will be applied in accordance with the GI and Biodiversity Supplementary Planning Document”

Without detailed proposals for mitigation and compensation, it is not possible to assess the development with regard to Policy NE7.

“Policy NE8 Protection of designated biodiversity and geodiversity sites

Planning permission for development which impacts on designated sites of importance for biodiversity and geodiversity, their features of interest and their role in the wider ecological network, directly or indirectly, will only be granted where:

a. the benefit of the development outweighs the significance of the protected site and its position in the hierarchy, and the harm caused

b. it can be demonstrated that avoidance and mitigation has been followed in accordance with Policy NE7 and relevant legislation

c. they are accompanied by a relevant assessment to demonstrate the impact of development upon the designated site.

This policy will be applied in accordance with the GI and Biodiversity Supplementary Planning Document*.

*to be written and consulted on prior to the local plan examination.”

The above clauses have not been demonstrated within the application, avoidance and mitigation have not been adequately demonstrated and as explained above, the ecological assessment is lacking in information.

Finally, On Page 171 of the Local Plan MDC explains why these policies are needed:

“9.62 These policies are required to prevent further significant losses in wildlife and habitats such as heathland and wildflower rich grasslands. This is addressed through a combination of: wider landscape-scale conservation measures linking designated sites with important habitats and landscape features outside these designations’

It is clear that there is an understanding within MDC concerning the vital importance of connectivity of landscape for wildlife and for the community as a whole. The application would be contrary to efforts for landscape scale conservation with significant negative impact on connectivity of the Maun corridor.

Summary

In summary, due to the clear contradiction with the Local Plan (both extant and draft) and the NPPF, failure to supply adequate ecological information, and lack of evidence to demonstrate avoidance, mitigation and compensation measures, NWT objects to this application.

The Maun Conservation Group strongly objects to the above-mentioned outline planning application.  We have a policy to “Protect, Promote and Preserve” the Quarry Lane Local Nature Reserves.

- We encourage the young to appreciate their local environment

- We recognise how important our green spaces are to our community

- We help Mansfield District Council to maintain the local nature reserve

- We have made improvements to the site for the benefit of the public

- We enable nature and wildlife to thrive in this “hidden gem” of a nature reserve.

Nature Reserve: (The access road will bisect the Quarry Lane Local Nature Reserve)

- A large area of the Quarry Lane Local Nature Reserve will be converted to concrete and tarmac and from then on it will not support biodiversity and there will no longer be any connectivity from one part of the reserve to the other for the wildlife.

- Prevents the movement of fauna through the Local Nature Reserve and its green corridor.

- Water Vole habitat, a protected and endangered species.  Movement and population will be damaged.

- White Clawed Crayfish, a protected and endangered species are very close to this development and may be put at risk.

- Bullhead fish, which supports aquatic birds, habitat, movement and population will be damaged.

- Prevents the public from traversing the Quarry Lane Local Nature Reserve

- Closing two footpaths, one of which is a public right of way across the Quarry Lane Local Nature Reserve

- Disrupting the movement of children, pedestrians, cyclists who use the site daily as a safe route.

- Making access across the reserve dangerous to people.

- Curtailing school visits, both during the construction phase (expected to be many years) and afterwards due to Health & Safety.

- Limiting access - The Maun Conservation Group work on the Local Nature Reserve with over 500 school children annually from numerous schools and other educational and community groups. These groups and others who continually use the site, throughout the year, will have limited or no access. Education is an important aspect of the work of the Maun Conservation Group.

- Construction work will have serious permanent negative impact on the Quarry Lane Local Nature Reserve with damage to birds, bats, water voles, fish, invertebrates, amphibians, the quality of the water and the community. There will be pollution of the water in the River Maun, the quality of which has improved in recent years,

- During construction and when complete – traffic noise, pollution and additional lighting will have serious permanent negative effects upon the Quarry Lane Local Nature Reserve, the wildlife and the community.

- A review of the RPS ecology survey submitted to the Mansfield District Council Planning Department and the findings of the Maun Conservation Group are below:

  - We welcome the survey work completed prior to outline planning application. The findings are broadly in line with our own observations. There are discrepancies, some of which will be the result of being of a “snapshot nature survey” whereas our observations run for many years and in all seasons.

  - The quarry was a major site for Great Crested Newts, However, a few years ago in a bulldozing of the site, the contractor filled many of the water filled pits, in what was the quarry floor. This act severely affected the GCN population. We feel that any development of this site should incorporate features that will allow the re-establishment and long-term maintenance of this severely endangered red list species. The four ponds in the proposed development may well lend themselves to this end.

  - One aspect of the report, which is at variance with our observations, is that only Smooth Newts are reported. Our observations find Palmate Newts are the more common species. We realise that in field observations of the young the two species are hard to differentiate.

  - Water voles are present and this is a delight to the Maun Conservation Group and locals when this shy creature is seen. The bridge development is unacceptably close to one of the most persistent locations for the species. We would not regard relocation as an acceptable option, unless it was within the nature reserve itself. This would probably require significant habitat improvement. Maun Conservation Group members do not want our children to miss seeing water voles – we want to protect the water voles and keep them on site.

  - Your own observations show four bat species in the vicinity - we have found five. Given this it is disappointing that with two proposed bridges being built no indication of building bat roosts into these is indicated. We also feel that if trees are removed nesting and roosting sites should have been included in the housing plans. A walk through Quarry Lane Nature Reserve on a summers evening reveals large numbers of bats present.

  - Mansfield District Council have encouraged and directed the installation of bat boxes on the Quarry Lane Local Nature Reserve in mitigation for other locally agreed planning developments. Where does Mansfield District Council expect bats to be relocated in the future?

  - The River Maun is a post-industrial water course which has successfully improved over the years. There is a good variety of fish species, and in particular a good population of bullhead. These in turn support the kingfishers and herons that are resident. We feel extremely privileged to have such “landmark species” in a town location. Whilst the bullhead is not as high priority as the red list species mentioned above. We feel that survey work on numbers and distribution of this species before and after work on the site would provide an indicator of environmental damage caused by the development. We feel that accountability for any depreciation in stock levels should be built into any conditions if development is approved.

  - In the period after the first survey we were involved in survey work on the White Clawed Crayfish with Notts County Council Biodiversity Action Group. We have shown a well-established breeding population in the Caudwell brook, within 500m of the development site. We believe the strength of the population is such that this must lead to some animals each year arriving in the River Maun.  We therefore feel it essential that proper survey work is conducted prior to any development in conjunction with Notts County Council Biodiversity Action Group.

  - We note that the original survey work is out of date as development did not start in spring this year, therefore new survey work is required. It would be appreciated by the Maun Conservation Group if the ecology consultants could keep us informed of any planned surveys.

  - The RPS ecology document needs further clarification as it is not always clear whether they are referring to Gregory’s Quarry or the Local Nature Reserve.  The two habitats are distinctly different and the officers and the planning committee may need additional assistance to appreciate which site is being referred to.  Latif Vajzovic BSc (Biological Sciences) MSc – member of the Maun Conservation Group

- There are significant caveats in the Nottinghamshire County Council Highways report.  It is clear that the Highways Department have significant concerns about this proposed development.  There are issues with regard to ownership of land around the proposed housing development.  There are restrictions with regard to the number of houses and flats and that the internal layout of properties should reflect these maximum numbers. Should outline planning progress any future modifications to the current proposal or any alterations to the original planned numbers of 114 houses and 86 flats should be refused, if numbers are increased.

- This development should not be given permission.  If it is it would be setting a precedent for future destruction of green and open spaces within the Mansfield area.

- The above demonstrates; the hundreds of local people, councillors etc who continue to actively support the nature reserve; the tens of thousands of pounds that have been invested; the support of dozens of local companies and the thousands of hours of conservation effort that has been invested for the long-term future of the reserve. All these people should be involved and represented, their motivations and intentions accounted for in the review of this planning application.

MDC Parks and Open Space

From a park departments prospective we are very concerned of the impact this development, if approved will have on Quarry Lane LNR. In essence the LNR will be divided in two which in our opinion will have massive implications for pedestrians, the friends group, schools who use the LNR in partnership with Maun Conservation Group and MDC. The safety of users will be compromised, the vehicle access and exits from the development across the existing LNR onto Quarry Lane itself will generate a huge amount of additional traffic. This coupled with the recent development, the business units adjacent to the LNR and the business park off Hamilton Road, all erected after the quarry finished quarrying will all contribute to excessive traffic problems on this, already busy and unsuitable road for this amount of traffic. The taking of part of the LNR for access reasons will completely spoil a very well loved and used Green Flag Award winning LNR and we find it quite disappointing that alternative access points have not been looked into in detail. We have no opposition to the development itself, only the blatant disregard for preserving the LNR at Quarry Lane in full.

Having looked at the plans, it is also apparent that little Open Space is being designed into this development, therefore if the application is granted and approved and goes ahead, we would expect the Open Space S106 contributions to be used off-site to mitigate the loss of part of the LNR which will be in two separate sections. Improvements to footpath infrastructure throughout, picnic areas and extensive improvements on the LNR would soften the blow but not the pain of losing this valuable, historic part of Mansfield.

Members of the Public

394 objections to the proposals have been received from members of the public, some of whom have written more than once, following the receipt of amended submissions from the applicant. The following provides an overview and summary of the main points raised:

- It would adversely affect the local wildlife

- Access route would endanger pedestrians

- Adverse impact on Local Nature Reserve

- Landscape impact

- Land is currently a green wedge

- Land is not wholly owned by the applicant

- Access details would destroy the flora &y fauna in the area

- Area used for recreation

- Loss of valuable local amenity

- No demand for housing on this site

- Site not allocated for housing

- Contrary to the Local Plan

- Overdevelopment

- No affordable housing

- Risk of rock fall

Two letters in support of the scheme have been received by the Council.

POLICY AND GUIDANCE

National Planning Policy Framework 2019

Section 2 – Achieving Sustainable Development

Paragraph 8 states that the planning system has 3 overarching objectives – economic, social and environmental.

Paragraph 11 – this seeks to ensure that proposals that accord with the development plan are approved without delay and where there are no relevant development plan policies or the policies which are the most important for determining the application are out of date granting permission, unless the application of the policies in the Framework that protect areas or assets of particular importance provides clear reason for refusing the development proposed or any adverse impacts of doing so would significantly and demonstrably outweigh the benefits when assessed against the policies in the Framework when taken as a whole.

The presumption in favour of sustainable development does not apply where the plan or project is likely to have a significant effect on a habitats site (either alone or in combination with other plans or projects), unless an appropriate assessment has concluded that the plan or project will not adversely affect the integrity of the habitats site (paragraph 177of the Framework).

Section 4 – Decision making

The National Planning Policy Framework (NPPF) stresses that pursuing sustainable development includes the conservation and enhancement of the natural environment (paragraphs 7, 9 and 17) and moving from a net loss to achieving net gains for biodiversity (paragraph 109).

Paragraph 47 states that planning permission should be determined in accordance with the development plan unless material considerations indicate otherwise.

Paragraph 54 states that local planning authorities should consider whether otherwise unacceptable development could be made acceptable through the use of conditions or planning obligations.

Paragraph 55 states that conditions should only be imposed where they are necessary, relevant to planning and to the development to be permitted, enforceable, precise and reasonable in all other respects.

Paragraph 91 states that decisions should aim to achieve healthy, inclusive and safe places which are both are safe and accessible and enable and support healthy lifestyles.

Paragraph 92 states that the LPA should provide the social, recreational and cultural facilities and services the community needs, planning policies and decisions should: a) plan positively for the provision and use of shared spaces, community facilities (such as local shops, meeting places, sports venues, open space’, etc.).

Paragraph 102 – states that transport issues should be considered at an early stage so that the potential impact on the highway network can be assessed and so that parking is addressed as an integral part of the scheme.

Paragraph 108 states that in assessing development proposals, applications should ensure appropriate opportunities to promote sustainable transport, safe and suitable access for all and any significant impact on the transport network is mitigated.

Paragraph 109 states that development should only be prevented or refused on highways grounds if there would be an unacceptable impact on highway safety, or the residual cumulative impacts on the road network would be severe.

Paragraph 127 – states that decisions should ensure developments;

a) will function well and add to the overall quality of the area, not just for the short term but over the lifetime of the development;

b) are visually attractive as a result of good architecture, layout and appropriate and effective landscaping;

c) are sympathetic to local character and history, including the surrounding built environment and landscape setting, while not preventing or discouraging appropriate innovation or change (such as increased densities);

d) establish or maintain a strong sense of place, using the arrangement of streets, spaces, building types and materials to create attractive, welcoming and distinctive places to live, work and visit;

e) optimise the potential of the site to accommodate and sustain an appropriate amount and mix of development (including green and other public space) and support local facilities and transport networks; and

f) create places that are safe, inclusive and accessible and which promote health and well-being, with a high standard of amenity for existing and future users and where crime and disorder, and the fear of crime, do not undermine the quality of life or community cohesion and resilience.

Adopted Mansfield Local Plan 2020

Policy S1 – Presumption in favour of sustainable development

Policy P7 – Amenity – states that development should be constructed to minimise impacts on the amenity of existing and future users

Policy IN2 – Green infrastructure - Development proposals within or adjoining areas of strategic green infrastructure (as shown on the Policies Map) will be supported

Policy IN3 – Protection of community open space and outdoor sports provision –

      - All areas of community open space shown on the policies map, outdoor sports provision identified for protection in Appendix 10, and any additional future provision made as part of new development will be safeguarded, and

      - Development proposals involving the loss of open space are required to provide an assessment of need, identifying proposed enhancement and/or replacement facilities, as relevant.

Policy IN6 – Designated local green space

1. Other than in very special circumstances, proposals will only be supported for development on a site designated as local green space, as shown on the Policies Map, where the development would clearly enhance or be ancillary to the reason/s why it was designated.

2. Development proposed within close proximity to a local green space will only be supported where it can be clearly and satisfactorily demonstrated that the development would not significantly harm the purpose/s for which the local green space was designated.

Policy IN9 – Impact of development on the transport network – states that development proposals will be supported provided that they do not endanger highway safety, and allow satisfactory access and egress from the highway and internal movements within the site.

Policy IN10 – Car and cycle parking – states that development proposals will be supported where there is appropriate provision for vehicle and cycle parking, including meeting the needs of the disabled. Provision should be designed so that it is an integral part of the development and does not dominate the public realm.

Policy H1 - Housing allocations - Gregory Quarry is not allocated in the Local Plan Policies for housing development

Policy NE2: Biodiversity and geodiversity – Development proposals will be supported where, commensurate with their scale, location and type, they:

a. protect, enhance and contribute to the management of the ecological network of habitats and sites of European, national and local importance (statutory and non-statutory);

b. avoid and/or minimise adverse individual and or cumulatively impacts, on biodiversity, geodiversity and ecosystem services;

c. seek to deliver a net gain in biodiversity across local and landscape scales; and

d. prioritise the de-fragmentation, restoration, retention and sensitive management of habitats and landscape features, to allow for the movement of wildlife.

Designated local sites - Development proposals will not be permitted where they will have a significant adverse impact on a designated local wildlife site, local nature reserve, or local geological site. Exceptions will only be made where the reasons for, and benefits of, the proposed development clearly outweigh the adverse impact on the loss or deterioration of the designated site.

Policy NE2 echoes the requirements of the NPPF, planning guidance and Section 40 of the NERC (2006), which places a duty on all local authorities to conserve biodiversity.

New development should also explore and implement solutions for preventing and reversing habitat fragmentation across local and landscape scales. This may include retaining and enhancing existing habitats and important landscape features on site, such as ponds and hedgerows.

An appropriate level of ecological assessment will be required to demonstrate how development proposals meet the requirements of this policy. Considering how new development can strengthen ecological networks is key to providing net gains in biodiversity whilst improving the natural environment's resilience to change. The Defra Biodiversity Metric and key principles, as set out in the ‘Biodiversity Net Gain: good practice principles for development 2016’ (see table 10.3), are key tools for assessing and demonstrating how net gains in biodiversity will be delivered and monitored through new development.

ISSUES

The main issues to consider in in the determination of this application relate to the following:

- The Principle of development

- Highway implications and whether there is enough detail to allow access to be determined

- The implications for biodiversity of the site and the surrounding area

- Developer Contribution

- Geo-technical Audit

- Other matters

1.	The Principle of development

Gregory Quarry is not allocated in the Local Plan Policies for housing development. Notwithstanding that the site was submitted for consideration in the call for sites during the preparation of the Mansfield Local Plan (2020), it was not considered suitable for allocation. The Local Plan was adopted in its current form despite the applicant’s challenge at the Local Plan Examination.

Appearance, landscaping, layout and scale of the proposal is not a matter for consideration in this outline planning application, given that the application relates to the proposed use and means of access only. An indicative plan was submitted with the application, but are not matters to be considered at this stage.

The quantum of the proposed development is likely to result in the inability to protect any of the woodland on the application site, and contrary to the desire of the ecology submission on behalf of the applicant, it is impossible to second guess any mitigation and/or compensation at this stage to demonstrate how ‘no net loss’ of biodiversity would be achieved. It is questioned how predictions in respect of biodiversity net gain can be ascertained now, given that no detailed information is available in respect of adverse impacts to areas or assets of particular importance, which would significantly and demonstrably outweigh the benefits when assessed against the policies in the Framework when taken as a whole. That this has not been provided and mitigates against a positive determination in its absence. The Local Planning Authority cannot realistically assess how the proposal meets requirements of the Framework and development plan. The scale and nature of the impact of the access to the proposed development site on the Local Nature Reserve, local wildlife sites and strategic green infrastructure network is unclear.

In assessing the development proposal, the Local Planning Authority considered whether appropriate opportunities were submitted to promote sustainable transport and safe and suitable access for all, and whether any significant impacts were mitigated. The conclusions were that the proposed development failed to meet the above criteria contrary to the provisions of paragraph 108 of the Framework.

2.	Highway implications and whether there is enough detail to allow access to be determined

The highway implications of the proposed development are outlined above in the consultation responses received from the Local Highway Authority. The comments are self-explanatory and suggests that there is insufficient detail to allow access to be determined now.

Concerns over the ability to provide the required status of the access for emergency access. Conflicting information regarding emergency accesses – legal opinion states that they must be adopted.

The Local Highway Authority considers that the access costs are grossly underestimated, which could prejudice the viability of the site access, and result in unsatisfactory access arrangements. These are discussed further below, under ‘Other Issues’.

The proposals are prejudicial to the existing users of the Mansfield Way by requiring them descend from this off-road route, cross a wide road in conflict with motorised vehicles and cyclists on the cycle route and then ascend either steps or a large ramp to return.  The principle of the proposal therefore appears to be in contravention of Paragraph 110 of the NPPF and a ‘like for like’ facility should have been provided in the form of a bridge.

However, this would involve crossing land which is not in the applicant’s control and if that option were to be pursued the applicant/developer would have to arrange a suitable Deed of Grant with all relevant adjacent land owners securing that access for NCC, so that there would be certainty that it would remain open in the future.  However, this introduces further uncertainty in respect of the delivery of the required accesses and another ransom potential and financial uncertainty.

Emergency accesses are not normally acceptable for numerous reasons, but without it, the proposed development is considered to be unacceptable, as it risks being completely cut off should the bridge be closed and this would also not address the core requirements of the NPPF in terms of sustainability - with the link, the development is around 400m distant from the nearest bus stop and without it circa 750 metres. The provision of an emergency access has its disadvantages, as in addition to emergency accesses not normally being acceptable (due to enforcement problems arising from their misuse, potential difficulties that could be encountered by the emergency services, maintenance issues and vandalism of access-control equipment, and general crime and anti-social behaviour problems) the applicant would not be in control of all of the land required to provide it.

Quarry Lane Access - This is a complex engineering project involving structural retaining walls and significant amounts of earthworks.  As the land drops away steeply to the south of Quarry Lane, the retaining wall will be large and visually significant.  It is considered that the costs of this in the viability assessment are significantly underestimated.

Furthermore, it is required to provide suitable construction access to the site, as this would not be allowed from the A60 (along the emergency access route).

3.	The implications for biodiversity of the site and the surrounding area

The Framework and Local Plan NE2 seek a net gain for biodiversity.  Planning Practice Guidance (PPG) defines biodiversity net gain as works which deliver “measurable improvements…  by creating or enhancing habitats in association with development.”  The application included survey data, much of which was collected in 2015 and is currently out of date.

To properly assess the application, it would be necessary for further detailed surveys to be undertaken prior to commencement to update the submitted information and to undertake missing/inadequate survey work undertaken in 2015. A small additional area to the north was subject to a walkover survey in 2017 and it is not clear whether the protected species survey work from 2015 covered this area. If detailed, up to date surveys do not cover the whole application area and surrounding zone of influence, there is a risk that ecological impacts may not have been adequately assessed.

Paragraph 175 of the Framework states that when determining planning applications, local planning authorities should apply the following principles:

a) if significant harm to biodiversity resulting from a development cannot be avoided (through locating on an alternative site with less harmful impacts), adequately mitigated, or, as a last resort, compensated for, then planning permission should be refused;

b) development on land within or outside a Site of Special Scientific Interest, and which is likely to have an adverse effect on it (either individually or in combination with other developments), should not normally be permitted. The only exception is where the benefits of the development in the location proposed clearly outweigh both its likely impact on the features of the site that make it of special scientific interest, and any broader impacts on the national network of Sites of Special Scientific Interest;

c) development resulting in the loss or deterioration of irreplaceable habitats (such as ancient woodland and ancient or veteran trees) should be refused, unless there are wholly exceptional reasons and a suitable compensation strategy exists; and

d) development whose primary objective is to conserve or enhance biodiversity should be supported; while opportunities to incorporate biodiversity improvements in and around developments should be encouraged, especially where this can secure measurable net gains for biodiversity.

The proposed access road would cause considerable damage to both the Quarry Lane LNR/River Maun corridor, and the Mansfield Way, severing a green corridor and introducing a considerable amount of disturbance (e.g. significant numbers of vehicle movements). Of particular concern to the County Council is the impact on the Mansfield Way, a section of former railway embankment which forms the northern boundary of the residential development and is managed by NCC Green Spaces.

The section of the Mansfield Way at Quarry Lane is a 0.5-mile-long permissive route linking the Mansfield Leisure Park off the A60 Nottingham Road with a public footpath running south-west from Mansfield town centre adjacent to the Robin Hood Railway Line.  This route includes the historic Drury Dam Viaduct, which supplies a valuable off-road crossing of Quarry Lane.  When extended to include the adjoining footpath the route also provides an off-road crossing of Sheepbridge Lane.

The access road to the proposed development as shown on Site Plan Proposed revision P08 dated 14/08/20 would isolate the section of path at the Mansfield Leisure Park (western) end from the Viaduct and both off-road crossings (western end).  A significant diversion though Quarry Lane LNR would be required to cross the access road and then follow a further diversion to gain access to the Viaduct.  Given that the Mansfield Way in this area is a valued and well-used off-road route this is not acceptable.  It should be noted that the disused railway line forming the Mansfield Way was purchased by Nottinghamshire County Council in 1988/89, meaning that permissive access has been provided for somewhere in the region of 30 years.

The Preliminary Ecology Appraisal (PEA) identifies that the site is well connected to the surrounding area and that it contains three designated sites, Quarry Lane Local Nature Reserve (LNR), River Maun Local Wildlife Site (LWS) and Maun Woodland and Scrub LWS. However, upon examination of the latest site plan, all three sites as well as the ecological connectivity are under threat from the proposed access road. The River Maun and associated habitats clearly provide an ecological corridor function which would be severely impacted by the current proposal.

The proposed access road which bisects the River Maun, Quarry Lane LNR and both LWSs appears to be in direct conflict with Policy NE2 Biodiversity of the Mansfield Local Plan (2020). It is considered therefore that the proposed access road is unacceptable on both ecological and planning policy grounds.

Surveys recorded the presence of amphibians including smooth newts, frogs and common toads (a NERC (2006) Sn 41 Species of Principal Importance). The PEA states that there are likely to be ‘minimal alternative breeding opportunities’ within the surrounding habitat. This seems to indicate that the loss of standing water on site would cause considerable impact on these species.

The habitat assessment for bats concluded that the site provides ‘High’ habitat quality. Therefore, in accordance with the stated guidelines (Collins, 2016) transect and static bat surveys undertaken are not considered adequate to fully assess the importance of the site for bats. A total of three transect surveys were carried out in June, July and September along with a single point of static monitoring on three occasions. According to Collins (2016), where a site is assessed as having ‘High’ habitat quality, the guideline is for up to two transect surveys per month (April to October) and three static monitoring locations per transect, again monthly (April to October). Along with additional transect surveys, there are a number of trees identified with moderate potential for bat roosting which require further survey work to be undertaken before this application is determined. Once the results and assessment of the above surveys are available, the applicant should demonstrate to the LPA how adverse impacts could be avoided, mitigated or compensated.

A thriving water vole population has been identified on the River Maun in the area of the proposed new access road/bridge. The development works would impact directly on water vole habitat during construction, potentially fragmenting the population. Post-construction, it is considered highly likely that impacts would continue due to disturbance from noise, additional traffic and lighting for example. In addition to severance of the valuable ecological corridor formed by the River Maun and Quarry Lane LNR, the access road is considered unacceptable due to impact on water vole, a species protected under the Wildlife and Countryside Act 1981 (as amended).

Breeding and wintering bird surveys were not undertaken, however the PEA identified suitable nesting habitat for a wide variety of bird species. Birds of particular interest known to the Nottinghamshire Wildlife Trust, include kingfisher, spotted flycatcher, woodcock, song thrush, mistle thrush, starling, dunnock, and mallard – all listed as birds of Conservation Concern (Red and Amber lists). For a proposed development of this scale on a site with such a wide variety of habitats, the Local Planning Authority would expect a proper breeding and wintering birds’ surveys to been undertaken in order to assess the potential impact on protected and priority bird species. This work should have been undertaken prior to the submission of the application.

Local authorities have statutory duties to deliver Water Framework Directive (Directive 2000/60/EC) objectives – this EU legislation is implemented in England and Wales though The Water Environment (Water Framework Directive) (England and Wales) Regulations 2003. Article 4(7) of the WFD sets the criteria that must be applied when determining development applications.

The proposal has the potential to have a negative impact on the water environment; however, no WFD assessment has been provided which could assist the LPA address this issue.

4.	Developer Contributions

The following sets out the Planning Obligations that are sought by Nottinghamshire County Council to mitigate the impact of the above development, if approved, based upon the amended plans.

Transport and Travel

A Bus Stop Infrastructure contribution of £12,500 is required to provide improvements to the two bus stops on Nottingham Road denoted as MA0034 and MA004, Alexandra Avenue and shall include real time bus stop poles & displays including associated electrical connections.

MA0034 Alexandra Avenue – Polycarbonate clear channel advertising bus shelter and raised boarding kerbs

MA0045 Alexandra Avenue – Polycarbonate clear channel advertising bus shelter and raised boarding kerbs

Sustainable Travel

Free introductory bus travel should be made available to residents of the development to encourage usage of public transport, which should be set out in the Travel Plan.

Justification

The level of funding requested would provide the following improvements:

MA0034 Alexandra Avenue – real time bus stop poles & displays including associated electrical connections

MA0045 Alexandra Avenue – real time bus stop poles & displays including associated electrical connections

These stops are serviced by a high frequency fully accessible daily bus service and the current facilities at these stops do not meet the standards as set out in the Appendix to the Council's Public Transport Planning Obligations Funding Guidance for Prospective Developers.

Waste Management

Considering the amended plans and the increase of dwellings from 200 to 204, a contribution for Mansfield Recycling Centre is £14,624.76 based upon 204 dwellings.

Education

Primary

The revised proposal for 204 dwellings would yield an additional 43 primary aged pupils within the Mansfield East Primary Planning Area. The County Council's previous consultation response to this application sought contributions towards the delivery of two new schools within the planning area. The County Council has delivered a one form entry expansion (equivalent to a new school) at King Edwards Primary School in order to ensure a sufficiency of places for pupils arising from new development in the area. The Council therefore wishes to maintain its requirement for contributions in order to retrospectively fund the expansion project which had been forward funded in anticipation of new housing. Based on the latest cost per pupil place for school expansions, the County Council would seek a contribution of £749,318 (£17,426 per place x 43 pupils).

Secondary

The revised proposal for 204 dwellings would yield an additional 33 secondary aged pupils within the Mansfield Secondary Planning Area. Based on current projections, there is currently insufficient capacity within the planning area to accommodate the additional pupils generated. Based on the latest cost per pupil place for school expansions, the County Council would seek a contribution of £787,875 (£23,875 per place x 33 pupils) towards additional secondary school provision in the Mansfield Secondary Planning Area.

Nottingham and Nottinghamshire Clinical Commissioning Group (CCG)

The CCG advises that the Orchard Medical Practice, St Peters Medical Practice, Millview Surgery and Acorn Medical Practice are the practices most likely to be affected by the development and has requested a contribution of £108, 375 in line with NPPF guidelines.

MDC Parks and Open Space

Have stated they would expect the Open Space (OS) S106 contributions to be used off site to mitigate the loss of part of the LNR which will be in two separate sections. Improvements to footpath infrastructure throughout, picnic areas and extensive improvements on the LNR would be required in line with NPPF guidelines.

Affordable Housing

The site lies within Zone 1 as identified within Appendix 6 of the Mansfield Local Plan 2020 and as such therefore 5% of housing provided would need to be affordable units.

5.	Geo-technical Audit

AECOM were commissioned in October 2020 to provide an independent geotechnical audit of submitted reports on the condition and stability of existing internal quarry slopes at the site. The report concludes inter alia, that a thorough desk based review of available geological information such as maps and memoirs, and assessments of any archival data on neighbouring development, and a more detailed inspection of the existing slopes, reported in accordance with the terminology and required information for a description of a rock mass set out in BS 5930:1999 Amendment 1 and BS EN ISO 14689 is deemed necessary.

This should be followed by a more structured and geologically plausible assessment of the anticipated stability risks and feasible mitigation measures on each of the main slope sections which will be exposed in the development around the quarry taking account of their differing orientations. An appraisal of groundwater conditions is also necessary at this stage.

The applicants responded to the above concerns by undertaking further inspection works which have been welcomed by AECOM, who go on to state the following;

“The letters from CODA and Silkstone contain reasonable proposals that commit the developer to complying with and discharging planning conditions at detailed planning application and consent stage, by way of providing investigations, designs and mitigation measures which deal with and resolve matters of slope stability. We note also that the letter from CMS dated 11 September 2020, sets out potential ways of managing and maintaining stable slopes throughout the completed quarry development which would rest with a management company, independent of the individual residents.”

It is considered that such an approach is acceptable, however there will still need to be a written agreement as to how the slopes would be inspected and maintained, the frequency of inspections etc, if planning consent was granted.

Other matters

Viability

Paragraph 57 of the NPPF states “Where up-to-date policies have set out the contributions expected from development, planning applications that comply with them should be assumed to be viable. It is up to the applicant to demonstrate whether particular circumstances justify the need for a viability assessment at the application stage. The weight to be given to a viability assessment is a matter for the decision maker, having regard to all the circumstances in the case, including whether the plan and the viability evidence underpinning it is up to date, and any change in site circumstances since the plan was brought into force. All viability assessments, including any undertaken at the plan making stage, should reflect the recommended approach in national planning guidance, including standardised inputs, and should be made publicly available.”

As part of this application, the applicant has submitted a Financial Viability Assessment which has been independently reviewed on the Council’s behalf by DVS Property Specialists dated 16 November 2020.

The report concluded that with 5% affordable housing and S106 costs amounting to £2,087,682, adopting a blended profit of 19.66% the scheme produces a residual land value amounting to £146,874 which equates to a rate of £13,149 per net acre. As this is below the suggested benchmark land value, the scheme is deemed to be not viable.

The Highway Authority remains of the view that accurate information in support of the viability assessment should be provided and this does not appear to be the case.  It is therefore considered that there is a significant risk that the development cannot provide the necessary access arrangements.

The following issues are challenged in the viability assessment:

- The access to the site – this now includes significant ramps and steps which are not included, but this may well be unacceptable to NCC in any case and require a bridge capable of taking maintenance vehicles.  The cost of the required Traffic Regulation Order has also not been included and whilst the report uses the estimate given in Appendix VII, the body of the report itself identifies that the box culvert used (costed at £275,000 would be more in the region of £650,000 (and this is considered still to be an underestimation).

- 80k has been allowed for offsite mitigation works. As £50k is required for the works to the traffic signals (as set out in the HTN) this leaves £30k for the widening works on Quarry Lane, which have actually been costed at £553,100 WITHOUT the required retaining structure, along with the proposed pedestrian refuge to allow safe pedestrian access to and from the bus stops on Nottingham Road costed at £8,100. £30k is clearly therefore a significant underassessment of costs.

- Commuted sums for the structures (bridge and retaining) and traffic calming have not been included and represent a significant cost.

The Local Highway Authority considers that the access costs are grossly underestimated, which could prejudice the viability of the site access, and result in unsatisfactory access arrangements.

Quarry Lane Access - This is a complex engineering project involving structural retaining walls and significant amounts of earthworks.  As the land drops away steeply to the south of Quarry Lane, the retaining wall will be large and visually significant.  We believe the costs of this in the viability assessment are significantly underestimated.  Many of the Local Highway Authority’s (LHA) concerns regarding viability are based on the unknown cost of acquisition of land/rights over land owned by 3 separate landowners.  Should development commence prior to this being built, there is a risk that houses could be built without suitable access being able to be achieved.

Following more detailed scrutiny of the submitted viability assessment the LHA continues to express significant concerns. For example, the figures quoted in the Highway Technical Notes do not correlate with the figures referred to in the Viability Assessment, which is uploaded on to the LPA’s website.  However, as the valuation of ransom strips are often based upon percentage of profit, the uncertainty and the LHA’s concerns remain.

The Highway Authority has drawn the LPA’s attention to the risks associated with basing the viability assessment upon uncertain costs, which could result in the inability to afford the access works required for the site.

It is not clear how the figure for acquisition of rights to access the development has been ascertained but the acquisition of rights to access land assumed at £500k is potentially unrealistic. The LHA has estimated this cost to be in the region of £2.6m - £4m given the presence of potential ransom strips.

The Technical Note suggests that commuted sums are covered in the valuation, but these have not been referred to in the viability report and there are no calculations.  Commuted sums will be required for the bridge, retaining structures, additional carriageway and potentially the ramp and steps to Mansfield Way. Currently, the value of these are unknown until the detail of the schemes are firmed up, but these are likely to be significant sums. A number of specific costs which have either been underestimated or not included in the valuation.

Whilst the viability assessment appears to include the development roads (“access roads), the figure presented appears to be a significant undervaluation.  The figure estimated for the roads and footways does not include the private drives and car parking as included in the viability assessment.  However, the estimate methodology does not allow a more accurate assessment to be made of the underestimation.   The estimate for the adoptable roads/footways is £750,000.  In the interests of being reasonable and to include for the private drives, we have suggested an underestimate of £500,000

Additional specific underestimations/costs not considered within the viability assessment are estimated to be in the region of £1,250,500.

CONCLUSION

The above analysis assesses the proposed development in light of the key issues highlighted as being material to the determination of the application.

In terms of Highway issues, there are shortfalls in the submission and there is a lack of certainty in terms of deliverability and viability. The consultation feedback is specific and relevant, and given that this outline application seeks consent for the access in addition to the principle of development, it is recommended that the concerns are taken into account in the determination of the application.

Not only is the application site not allocated in the development plan for residential development, having failed to be included in the Local Plan process and failed to persuade the Planning Inspector at the Examination stage that it was worthy of allocation.

Not only are there significant highway reasons why the development should not be approved, but there are also compelling biodiversity reasons why the development should not be permitted. The proposed access road would cause considerable damage to both the Quarry Lane LNR/River Maun corridor, and the Mansfield Way, severing a green corridor and introducing a considerable amount of disturbance (e.g. significant numbers of vehicle movements). Of particular concern to the County Council is the impact on the Mansfield Way, a section of former railway embankment which forms the northern boundary of the residential development and is managed by NCC Green Spaces.

The access road to the proposed development as shown on Site Plan Proposed revision P08 dated 14/08/20 would isolate the section of path at the Mansfield Leisure Park (western) end from the Viaduct and both off-road crossings (western end).  A significant diversion though Quarry Lane LNR would be required to cross the access road and then follow a further diversion to gain access to the Viaduct.  Given that the Mansfield Way in this area is a valued and well-used off-road route this is not acceptable.  It should be noted that the disused railway line forming the Mansfield Way was purchased by Nottinghamshire County Council in 1988/89, meaning that permissive access has been provided for somewhere in the region of 30 years.

The PEA identifies that the site is well connected to the surrounding area and that it contains three designated sites, Quarry Lane Local Nature Reserve (LNR), River Maun Local Wildlife Site (LWS) and Maun Woodland and Scrub LWS. However, upon examination of the latest site plan, all three sites as well as the ecological connectivity are under threat from the proposed access road. The River Maun and associated habitats clearly provide an ecological corridor function which would be severely impacted by the current proposal.

RECOMMENDED REASONS FOR REFUSAL

(1)

Paragraph 57 of the NPPF states “Where up-to-date policies have set out the contributions expected from development, planning applications that comply with them should be assumed to be viable. It is up to the applicant to demonstrate whether particular circumstances justify the need for a viability assessment at the application stage. The weight to be given to a viability assessment is a matter for the decision maker, having regard to all the circumstances in the case, including whether the plan and the viability evidence underpinning it is up to date, and any change in site circumstances since the plan was brought into force. Having reviewed the Viability Assessment It is considered that insufficient information has been submitted to demonstrate the deliverability and viability of the proposed access road. As such the proposal is considered contrary to Policy IN9 of the Adopted Mansfield Local Plan (2020) and the National Planning Policy Framework (2019).

(2)

The National Planning Policy Framework is clear that if significant harm to bio-diversity cannot be avoided, adequately mitigated or compensated, then planning permission should be refused. Policy IN6 (Designated local green space) of the Mansfield Local Plan (2020) indicates that other that in very special circumstances, proposals will only be supported for development on a site designated as local green space, as shown on the Policies Map, where the development would clearly enhance or be ancillary to the reason/s why it was designated. Furthermore it goes on to state that  development proposed within close proximity to a local green space will only be supported where it can be clearly and satisfactorily demonstrated that the development would not significantly harm the purpose/s for which the local green space was designated. The net impact of the development is unclear and a precautionary approach is thus appropriate in determining this application. The proposal is therefore not considered to be in accordance with Policy NE2, P7, IN3 and IN6 of the Mansfield Local Plan 2020 and the National Planning Policy Framework 2019.

(3)

Paragraph 127 of the National Planning Policy Framework States that Planning Policies should ensure that development will function well and add to the overall quality of the area, are visually attractive as a result of good architecture, layout and appropriate and effective landscaping, are sympathetic to local character and history, including the surrounding built environment and landscape setting and establish or maintain a strong sense of place, using the arrangements of streets, spaces, building types and materials.    In this case the proposed residential development would significantly harm the Quarry Lane Local Nature Reserve (LNR), River Maun Local Wildlife Site (LWS) and Maun Woodland and Scrub (LWS), through development of the proposed access road. Overall the incongruous layout of the proposal set within a wooded local nature reserve and wildlife site would significantly harm the character and appearance of the area and be at odds with the pattern of built form in the vicinity.